! ACC_1 : VTP , ethernet channel , trunk , vlan

en

conf t

host ACC_1

vtp domain cisco.com

vtp pass cisco123

vtp mode client

int r f0/21-22

channel-pro lacp

channel-gr 1 mode act

int por 1

sw mo trunk

int r f0/23-24

channel-pro lacp

channel-gr 2 mode act

int por 2

sw mo trunk

int r f0/1-10

sw mo acc

sw acc vl 10

int r f0/11-20

sw mo acc

sw acc vl 20

! ACC_2 : vtp , ethernet channel , vlan , trunk

en

conf t

host ACC_2

vtp domain cisco.com

vtp pass cisco123

vtp mode client

int r f0/21-22

channel-pro lacp

channel-gr 1 mode act

int por 1

sw mo trunk

int r f0/23-24

channel-pro lacp

channel-gr 2 mode act

int por 2

sw mo trunk

int r f0/1-10

sw mo acc

sw acc vl 30

int r f0/11-20

sw mo acc

sw acc vl 40

! ACC_3 : vtp , ether channel , vlan , trunk

en

conf t

host ACC_3

vtp domain cisco.com

vtp pass cisco123

vtp mode client

int r f0/21-22

channel-pro lacp

channel-gr 1 mode act

int por 1

sw mo trunk

int r f0/23-24

channel-pro lacp

channel-gr 2 mode act

int por 2

sw mo trunk

int r f0/1-10

sw mo acc

sw acc vl 50

int r f0/11-20

sw mo acc

sw acc vl 60

! DIS_1 : IP routing , vtp , trunk , ether channel , inter vlan + HSRP   , routing (ospf )

en

conf t

host DIS_1

ip routing

int g0/1

no sw

ip add 100.0.50.2 255.255.255.252

no sh

int g0/2

no sw

ip add 100.0.40.2 255.255.255.252

no sh

vtp domain cisco.com

vtp pass cisco123

vtp mode server

int r f0/1-2

channel-pro lacp

channel-gr 1 mode act

ex

int por 1

sw trunk en dot

sw mo trunk

ex

int r f0/3-4

channel-pro lacp

channel-gr 2 mode  act

ex

int por 2

sw trunk en dot

sw mo trunk

ex

int r f0/5-6

channel-pro lacp

channel-gr 3 mode act

ex

int por 3

sw trunk en dot

sw mo trunk

ex

vl 10

vl 20

vl 30

vl 40

vl 50

vl 60

ex

! HSRP ( active cho vl 10,20,30 )  ( stanby cho vl 40 ,50,60 )

int vl 10

ip add 192.168.10.2 255.255.255.0

stan 10 ip 192.168.10.1

stan 10 pri 200

stan 10 preem

ex

int vl 20

ip add 192.168.20.2 255.255.255.0

stan 20 ip 192.168.20.1

stan 20 pri 200

stan 20 preem

ip help 192.168.10.5

ex

int vl 30

ip add 192.168.30.2 255.255.255.0

stan 30 ip 192.168.30.1

stan 30 pri 200

stan 30 preem

ip help 192.168.10.5

ex

int vl 40

ip add 192.168.40.2 255.255.255.0

stan 40 ip 192.168.40.1

stan 40 pri 100

stan 40 preem

ip help 192.168.10.5

ex

int vl 50

ip add 192.168.50.2 255.255.255.0

stan 50 ip 192.168.50.1

stan 50 pri 100

stan 50 preem

ip help 192.168.10.5

ex

int vl 60

ip add 192.168.60.2 255.255.255.0

stan 60 ip 192.168.60.1

stan 60 pri 100

stan 60 preem

ip help 192.168.10.5

ex

router ospf 1

router-id 1.1.1.1

net 192.168.10.0 0.0.0.255 a 0

net 192.168.20.0 0.0.0.255 a 0

net 192.168.30.0 0.0.0.255 a 0

net 192.168.40.0 0.0.0.255 a 0

net 192.168.50.0 0.0.0.255 a 0

net 192.168.60.0 0.0.0.255 a 0

net 100.0.40.0 0.0.0.3 a 0

net 100.0.50.0 0.0.0.3 a 0

passive-interface vlan 10

passive-interface vlan 20

passive-interface vlan 30

passive-interface vlan 40

passive-interface vlan 50

passive-interface vlan 60

! DIS_2 : ip , vtp , ether channel , inter vlan + HSRP ,trunk , ospf

en

conf t

host DIS_2

ip routing

int g0/1

no sw

ip add 100.0.60.2 255.255.255.252

no sh

int g0/2

no sw

ip add 100.0.30.2 255.255.255.252

no sh

vtp domain cisco.com

vtp pass cisco123

vtp mo client

int r f0/1-2

channel-pro lacp

channel-gr 1 mode act

int por 1

sw trunk en dot

sw mo trunk

int r f0/3-4

channel-pro lacp

channel-gr 2 mode act

int por 2

sw trunk en dot

sw mo trunk

int r f0/5-6

channel-pro lacp

channel-gr 3 mode act

int por 3

sw trunk en dot

sw mo trunk

int vl 10

ip add 192.168.10.3 255.255.255.0

stan 10 ip 192.168.10.1

stan 10 pri 100

stan 10 preem

int vl 20

ip add 192.168.20.3 255.255.255.0

stan 20 ip 192.168.20.1

stan 20 pri 100

stan 20 preem

int vl 30

ip add 192.168.30.3 255.255.255.0

stan 30 ip 192.168.30.1

stan 30 pri 100

stan 30 preem

int vl 40

ip add 192.168.40.3 255.255.255.0

stan 40 ip 192.168.40.1

stan 40 pri 200

stan 40 preem

int vl 50

ip add 192.168.50.3 255.255.255.0

stan 50 ip 192.168.50.1

stan 50 pri 200

stan 50 preem

int vl 60

ip add 192.168.60.3 255.255.255.0

stan 60 ip 192.168.60.1

stan 60 pri 200

stan 60 preem

router ospf 1

router-id 5.5.5.5

net 192.168.10.0 0.0.0.255 a 0

net 192.168.20.0 0.0.0.255 a 0

net 192.168.30.0 0.0.0.255 a 0

net 192.168.40.0 0.0.0.255 a 0

net 192.168.50.0 0.0.0.255 a 0

net 192.168.60.0 0.0.0.255 a 0

net 100.0.30.0 0.0.0.3 a 0

net 100.0.60.0 0.0.0.3 a 0

passive-interface vlan 10

passive-interface vlan 20

passive-interface vlan 30

passive-interface vlan 40

passive-interface vlan 50

passive-interface vlan 60

! Core_1

en

conf t

host Core_1

ip routing

int g0/1

no sw

ip add 100.0.50.1 255.255.255.252

no sh

int g0/2

no sw

ip add 100.0.30.1 255.255.255.252

no sh

int f0/1

no sw

ip add 100.0.10.2 255.255.255.252

no sh

router ospf 1

router-id 2.2.2.2

net 100.0.10.0 0.0.0.3 a 0

net 100.0.30.0 0.0.0.3 a 0

net 100.0.50.0 0.0.0.3 a 0

! Core_2

en

conf t

host Core_2

ip routing

int g0/1

no sw

ip add 100.0.60.1 255.255.255.252

no sh

int g0/2

no sw

ip add 100.0.40.1 255.255.255.252

no sh

int f0/1

no sw

ip add 100.0.20.2 255.255.255.252

no sh

router ospf 1

router-id 4.4.4.4

net 100.0.20.0 0.0.0.3 a 0

net 100.0.40.0 0.0.0.3 a 0

net 100.0.60.0 0.0.0.3 a 0

! R1

en

conf t

host R1

int g0/0

ip add 100.0.10.1 255.255.255.252

ip nat in

no sh

int g0/1

ip add 100.0.20.1 255.255.255.252

ip nat in

no sh

int s0/0/0

ip add 200.200.10.1 255.255.255.252

ip nat out

no sh

ip route 0.0.0.0 0.0.0.0 200.200.10.2

router ospf 1

router-id 3.3.3.3

net 100.0.20.0 0.0.0.3 a 0

net 100.0.10.0 0.0.0.3 a 0

default-information originate

! NAT

acc 100 per ip 192.168.10.0 0.0.0.255 any

acc 100 per ip 192.168.20.0 0.0.0.255 any

acc 100 per ip 192.168.30.0 0.0.0.255 any

acc 100 per ip 192.168.40.0 0.0.0.255 any

acc 100 per ip 192.168.50.0 0.0.0.255 any

acc 100 per ip 192.168.60.0 0.0.0.255 any

ip nat in sou list 100 interface s0/0/0